DHY28KSE

| Model | DHY28KSE |
| --- | --- |
| Rated/Prime kVA* | 25 |
| Rated/Prime kW | 20 |
| Max/Standby kVA* | 28 |
| Max/Standby kW | 22 |
| Phase | Three |
| Voltage | 230 / 400 |
| Connection Method(s) | 2 x 230V/32A 1 x 230V/63A Busbar |
| Rated Speed (RPM) | 1500 |
| Frequency (Hz) | 50 |
| Max/Continuous Current (Amps) | 40 / 36 |
| Power Factor (Cosφ) | 0.8 |
| Fuel Type | Diesel |
| Fuel Tank (Litres) | 93 |
| Runtime (Hours) @ 100% Load | 10 |
| Noise Level @ 1m 75% load (dB) | 72 |
| Noise Level @ 7m 75% load (dB) | 67 |
| Hyundai Diesel Engine Model | HY4102 |
| Power Output (kw) | 33 |
| Displacement (CC) | 3800 |
| No. of Cylinders | 4 |
| Displacement (Litres) | 3.8 |
| Recommended Oil Type | SAE 15W40 |
| Oil Capacity (Litres) | 7.5 |
| Oil Filter Part No. | JX0814 |
| Aspiration | Natural |
| Fuel Regulation/Governor System | Mechanical |
| Fuel Consumption (100% Load) [Litres] | 9.3 |
| Fuel Filter Part No. | CX0708 |
| Cooling | Water |
| Alternator Model | 184F |
| Breakers | 3P+N C63A 100mA |
| Brushless | Yes |
| Voltage Regulation | AVR |
| Air Filter Part No. | K1317 |
| Net Weight (kg) | 870 |
| Gross Weight (kg) | 870 |
| Dimensions L x W x H (mm) | 2200 x 950 x 1250 |
| Battery System / Starting (V - Ah) | 1 x 12V - 60Ah |
| ATS / MTS Compatible | Yes |
| Control Panel | ComAp MRS10 |
| Accessories | - |
| Model EAN | 700621496236 |
| Warranty | 1 Year / 1000 Hours (can extended to 2 years, subject to standby use of 500 hours per year. Terms apply*) |