Questionnaire

High pressure pumping unit up to 1200 bar

| Customer FE Sanoat Energetika Guruhi LLC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address: Nukus Street 50, Tashkent, 100100, Republic of Uzbekistan |  |  |  |  |  |  |  |  |
| SECTION 1. Pump and pumping unit |  |  |  |  |  |  |  |  |
| Purpose of the acquisition. The purpose of the pumping unit ( hydro cleaning , pumping, pressure maintenance, pressure testing etc. - fill in in detail): |  |  |  | Water injection into the well annulus |  |  |  |  |
| Working mode |  |  |  | Constant |  |  | Yes |  |
|  |  |  |  | Periodic |  |  | - |  |
| Expected operating time, hour |  |  |  | Per day |  |  | 24 |  |
|  |  |  |  | In a year |  |  | 8000 |  |
| Pump type |  |  |  | Type T (built-in gearbox): |  |  | Yes |  |
|  |  |  |  | Type T (external gear): |  |  | - |  |
| Required nominal capacity, l/min (m3/h) |  |  |  | 40-70 l/min (4.2 m3/h) |  |  |  |  |
| Required nominal pressure, MPa |  |  |  | 120 |  |  |  |  |
| Adjustment | Pressure |  | Yes | Device type | Mechanical |  |  | Yes |
|  |  |  |  |  | Pneumatic |  |  | - |
|  |  |  |  |  | Electric |  |  | - |
|  |  |  | Not | - |  |  |  |  |
|  | Innings |  | Yes | Turns EM frequency converter | Delivery range (min-max) l/min |  |  | 40-70 l/min |
|  |  |  | Not |  |  |  |  |  |
| Suction pressure ( specify minimum/maximum) |  |  |  | Min: atm Max: 0.2 kg s/cm2 |  |  |  |  |
| type of drive |  |  |  | electric motor |  |  | Yes |  |
|  |  |  |  | ICE |  |  | Not |  |
|  |  |  |  | Other |  |  | Not |  |
| Execution |  |  |  | Stationary | Not |  | For block container | No |
|  |  |  |  | On a mobile trolley | Not |  | On a small trailer | Yes |
|  |  |  |  | On a wheeled chassis | Not |  | Sled | Not |
|  |  |  |  | Other: |  |  |  |  |
| Drive power limitations, kW |  |  |  | 160 kW |  |  |  |  |
| Necessity of explosion protection (for installations with electric motor) |  |  |  | Not |  |  |  |  |
|  |  |  |  | Hazardous area class: |  |  |  |  |
| Control of the parameters of the pumping unit (see Section 2) |  | ICE (diesel) |  | Local (manual) from the control panel: |  |  |  | No |
|  |  | electric motor |  | Local (manual) from the control panel: |  |  |  | No |
|  |  |  |  | Semi-automatic from control panel: |  |  |  | No |
|  |  |  |  | Semi-automatic from the control station: |  |  |  | Yes |
|  |  |  |  | Automatic from the control station |  |  |  | No |
| Pumped medium: fresh water |  |  |  |  |  |  |  |  |
| Density, g/cm3 |  | 1.07 |  | HDTV size, microns: |  |  | 200 |  |
| Kinematic viscosity cSt (mm2/s) |  | 1006 |  | The content of solid suspended particles of high-frequency solids,% by weight: |  |  | 2 |  |
| pH value, units |  | 6-8 |  | Additional requirements: |  |  |  |  |
| Required accessories: |  |  |  |  |  |  |  |  |
| Liquid filter; the degree of filtration is not worse than 200 microns |  |  |  | Low-pressure hose for suction (specify Du - Rn , length, type of connection); Yes |  |  |  |  |
| Suction pulsation damper |  |  |  | High-pressure hose for suction (specify Du - Rn , length, type of connection); Yes |  |  |  |  |
| SPTA for the service life, month |  |  |  | 24 |  |  |  |  |
| Other |  |  |  | High pressure hoses 3/8” 1800 bar - 1000 meters, fittings (plugs 20 pcs, distribution manifold 4 pcs, tees 10 pcs) |  |  |  |  |
| SECTION 2. Control of the parameters of the pumping unit (select the required one) |  |  |  |  |  |  |  |  |
| Control system required |  |  |  | Yes |  |  |  |  |
|  |  |  |  | - |  |  |  |  |
| Manual control mode for ICE installations |  |  |  | Control panel: diesel start/stop; manual speed control; mechanical or pneumatic bypass; pump and drive safety sensors (relay) |  |  |  |  |
|  |  |  |  | Other |  |  |  |  |
| Manual control mode for installations with EM |  |  |  | Control board: start/stop of the unit; soft start relay, star-delta; bypass mechanical or pneumatic; pump safety sensors (relay) |  |  |  |  |
|  |  |  |  | Control board: start/stop of the unit; electronic soft start; bypass mechanical or pneumatic; pump safety sensors (relay) |  |  |  |  |
|  |  |  |  | Control board: start/stop of the unit; frequency converter; bypass mechanical or pneumatic; pump safety sensors (relay) |  |  |  |  |
|  |  |  |  | Other |  |  |  |  |
| Semi-automatic control mode for installations with EM |  |  |  | Control station: start/stop of the plant; controller; frequency converter; bypass electric or pneumatic; relay sensors or electronic pump safety sensors |  |  |  | Yes |
|  |  |  |  | Other |  |  |  |  |
| Automatic control mode for installations with EM |  |  |  | Control station: start/stop of the plant; controller; frequency converter; bypass electric or pneumatic; electronic pump safety sensors; remote control of work parameters |  |  |  |  |
|  |  |  |  | Other |  |  |  |  |
| Additional requirement: |  |  |  |  |  |  |  |  |
| SECTION 3. Station control of the parameters of the pumping unit |  |  |  |  |  |  |  |  |
| 3.1. Energy supply |  |  |  |  |  |  |  |  |
| 0.4 kV |  |  |  | Yes |  |  |  |  |
| 6 kV |  |  |  | - |  |  |  |  |
| Other |  |  |  | - |  |  |  |  |
| electricity meter |  |  |  | brand |  |  |  |  |
|  |  |  |  | No |  |  |  |  |
| 3.2. Controlled parameters (indicate the types of sensors required, for example: relay sensor; electronic analog with a current output of 4 ... 20 mA etc. ) |  |  |  |  |  |  |  |  |
| Type of sensors |  |  |  | relay sensor |  |  | Yes |  |
|  |  |  |  | Analog 4…20 mA |  |  | - |  |
|  |  |  |  | Other |  |  | - |  |
| Fluid pressure and supply pipe |  |  |  | Before filter |  |  |  |  |
|  |  |  |  | After filter |  |  | Must be installed (safety sensor) |  |
|  |  |  |  | Filter differential pressure |  |  | Yes |  |
|  |  |  |  | Other |  |  |  |  |
| Discharge pressure |  |  |  | Yes |  |  |  |  |
| Reducer oil pressure |  |  |  | Yes |  |  |  |  |
| Reducer oil temperature |  |  |  | Yes |  |  |  |  |
| Pump oil pressure |  |  |  | Must be installed (safety sensor) |  |  |  |  |
| Pump oil temperature |  |  |  | Must be installed (safety sensor) |  |  |  |  |
| EM stator temperature |  |  |  |  |  |  |  |  |
| Bearing temperature ED working side |  |  |  |  |  |  |  |  |
| Bearing temperature ED field side |  |  |  |  |  |  |  |  |
| room temperature |  |  |  |  |  |  |  |  |
| Load current |  |  |  | Yes |  |  |  |  |
| Power consumption |  |  |  | Yes |  |  |  |  |
| flow meter |  |  |  | Yes |  |  |  |  |
| vibration sensor |  |  |  |  |  |  |  |  |
| Gas contamination of the room |  |  |  |  |  |  |  |  |
| Other Options |  |  |  |  |  |  |  |  |
| 3.3. Life support management |  |  |  |  |  |  |  |  |
| Electric heating control |  |  |  | Yes | By agreement |  |  |  |
| Ventilation control |  |  |  | Yes | By agreement |  |  |  |
| Additional requirements: |  |  |  |  |  |  |  |  |
| 3.4. Control system |  |  |  |  |  |  |  |  |
| Type, brand of controller |  |  |  |  | By agreement |  |  |  |
| 3.5. system equipment |  |  |  |  |  |  |  |  |
| Frequency converter |  |  |  | Yes | By agreement |  |  |  |
| Soft starter |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |
| Transfer of information to the control room |  |  |  |  |  |  |  |  |
| The need to communicate information |  |  |  | Yes |  |  |  |  |
| 3G , 4g, 5G, ( choose ) |  |  |  | 4G |  |  |  |  |
| Interface used |  |  |  |  |  |  |  |  |
| Supply valve control |  |  |  |  |  |  |  |  |
| Discharge valve control |  |  |  |  |  |  |  |  |
| Third party device management (list) |  |  |  |  |  |  |  |  |
| Additional requirement: |  |  |  |  | By agreement |  |  |  |
| SECTION 4. Visual inspection devices |  |  |  |  |  |  |  |  |
| Manometer vibration-resistant oil pressure 0 ... 1 MPa |  |  |  | Mandatory installed |  |  |  |  |
| Vibration-resistant pressure gauge for liquid pressure 0…1.6 MPa |  |  |  | Mandatory installed |  |  |  |  |
| Vibration-resistant pressure gauge for liquid injection 0 ... 1.6 MPa |  |  |  | Mandatory installed |  |  |  |  |
| Additional devices |  |  |  | By agreement |  |  |  |  |