PSA PRISM® NITROGEN PRODUCTION UNIT:

4" VALVE – SWITCH

TYPE OF EQUIPMENT: Valve

APPLICATION: In the cyclic operation diagram of the PSA unit

TECHNICAL DATA:

Valve:                                                                                                                                                                                  Size: 4" (100 mm)                                                                                                                                                              Installed between: 150# flanges (with raised seating surface)                                                                                Full open flow coefficient Cv: 400 gallons of water at 1 psi pressure drop and 60°F temperature                  Class: 150#                                                                                                                                                                         Type: Wafer type                                                                                                                                                              Body material: ASTM A216 type WCB                                                                                                                           Plug material: 316 SS (stainless steel)                                                                                                                            Stem material: 316 SS (stainless steel)                                                                                                                         Seat design: Teflon (PTFE)                                                                                                                                               Seals: Stem seals installed during assembly                                                                                                                Mounting size: 2.13" (54 mm)                                                                                                                                        Complete with flanges                                                                                                                                                                                                                                                                          Note: Includes seat -1 and seal kit- 1

ACTUATING MECHANISM:

Type: Double acting                                                                                                                                                          Inlet port size: 1/4" (6 mm)                                                                                                                                             Installable position: Cylinder axis parallel to the pipeline                                                                                         When air supply is stopped, the valve must: Remain in its last position                                                                Minimum air pressure required: 60 psi (4.2 kg/cm² gauge pressure)                                                                   Maximum allowable working pressure of entire mechanism: 150 psi (10.5 kg/cm² gauge pressure)            Note: Includes seals and gaskets such as O-rings and stem seals

SOLENOID VALVE:

SOLENOID VALVE:                                                                                                                                                             Channel size: 1/4" (6 mm)                                                                                                                                               Cv coefficient for valve: 1.1                                                                                                                                             Body material: Zinc, die-cast                                                                                                                                          Valve diaphragm material: Ceramic flat distributors (Al₂O₃)                                                                                     Power requirements: 120 V, 50/60 Hz                                                                                                                         Fail position: Closed                                                                                                                                                         Working pressure: 0 to 150 PSIG (0 to 10.5 kg/cm² gauge)                                                                                      Temperature range: 6 to 150 degrees F (–15 to 65.5 degrees C)                                                                             Drain plate 1/4") (6 mm)                                                                                                                                         Cable entry connector (1/2") (12 mm)

PROCESS PARAMETERS:

Pressure:                                                                                                                                                                             Normal: 120 (8.4)                                                                                                                                                              Max: 150 (10.5)                                                                                                                                                                 Min: 0 (0)                                                                                                                                                                            Unit: psi (kg/cm² gauge pressure)                                                                                                                               Differential pressure:                                                                                                                                                       Normal: 120 (8.4)                                                                                                                                                             Max: 150 (10.5)                                                                                                                                                                Min: 0 (0)                                                                                                                                                                            Unit: psi (kg/cm²)                                                                                                                                                              Temperature:                                                                                                                                                                     Normal: 80 (26.6)                                                                                                                                                             Max:104 (40)                                                                                                                                                                              Min: −20 (−29)                                                                                                                                                                   Unit: degrees F (C)

Quantity: 8 pcs.                                                                                                                                                                 TAG numbers: 62–1V550, 2V550, 1V553, 2V553,                                                                                                       62–1XV550, 2XV550, 1XV553, 2XV553